ABE 598 Project Team due 9/13/26

Project Members:

Emery Brewer – M.Eng Digital Agriculture

Junjie Ao

Proposed: Kinjalk Parth

Team Name: CoverageVision AI

Proposed Project Title: “AI-Driven Closed-Loop Precision Spraying for Real-Time Application Optimization”

Project Proposal due 9/21/26

### Project Proposal (10%)

- 30 pts: Problem definition & relevance

- 20 pts: Expected outcomes & success criteria

- 20 pts: Clarity and structure

- 30 pts: Technical approach & feasibility. Include:

  - team member responsibilities;

  - collaboration tools and methods;

  - data/software/hardware dependencies;

  - how team members will contribute;

  - expected deliverables from each member.

CoverageAI Project Proposal

Problem Definition:

Agricultural spray applications are influenced by both environmental and machine conditions such as wind speed and direction, humidity, temperature, vehicle speed, spray pressure, flow rate, and nozzle configurations. These factors can affect spray coverage, deposition, uniformity, and off-target movement. Spray systems typically operate using predefined settings or rely on feedback collected after an application has already occurred. While post-spray sensing can identify poor application performance, there is an inherent delay: by the time an application issue is detected, the sprayer has already moved beyond that area.

This project explores a conceptual AI-enabled spray control system that combines predictive pre-spray sensing with post-spray verification. Pre-spray environmental, machine, and application information would be used to predict appropriate spray parameters before application. After spraying, sensing, or imaging would evaluate the resulting performance and provide feedback on adjustments for application improvement.

Importance:

Improving spray application decisions before a product is applied would help agricultural sprayers respond more effectively to changing operating and environmental conditions. The proposed approach also creates the foundation for a more adaptive spray system that continuously learns from the relationship between operating conditions, selected spray parameters, and measured application outcomes. Such a system could support improved application consistency, reduced off-target losses, more efficient use of agricultural inputs, and better overall spray performance.

Objectives:

Technical Approach

Emery B.

Expected outcomes and success:

- Create a working demonstration of the system, whether it is simulation or applied to real data.

- Collected data from several sensors and parameters to have a predictive model on how that spray will perform.

- Have a validation or verification measurement tool for defining the spray output, potentially on a percentage scale. For example, a system could classify 30% or greater as acceptable, 15-30% coverage as marginal, and less than 15% as critical.

- Have a closed feedback loop to make adjustments as application is happening.

Technical approach and feasibility:

- Sensor input

  - Machine data (speed, boom height)

  - Environment (Wind speed/direction, temperature, relative humidity)

  - Spray system (Nozzle type, pressure, flow rate)

  - Application requirement (desired coverage/ deposition threshold)

- AI Model: The predictive model estimates expected spray performance for a given combination of environmental conditions and operating parameters.

- Parameter-selection step: Which available configuration is predicted to satisfy the application requirement under the current conditions?

  - Optimization problem. Meet required coverage while minimizing drift risk and unnecessary application.

- Post Spray Verification

- Feedback/Model update/ Dynamic Changes

Responsibilities and Contribution

Skills: Systems engineering, application technology, and agronomy.

Collaboration Method: Teams/Microsoft, biweekly meetings

Junjie A.

Expected outcomes and success:

- Develop a predictive machine learning model that estimates spray performance from pre-spray environmental, machine, and spray-system conditions.

- Establish a usable data pipeline for model development using real experimental data when available, with public or simulated data as a fallback.

- Evaluate the predictive model by comparing predicted spray performance with measured or simulated outcomes.

- Integrate the predictive model with the parameter-selection and post-spray feedback components of the overall system.

- Demonstrate that the model responds appropriately to changes in environmental conditions and spray parameters.

Technical approach and feasibility:

- Data acquisition and preparation

  - Use real experimental spray data if available through university laboratories, team resources, or collaborators.

  - Supplement with publicly available datasets and published experimental results.

  - If sufficient experimental data are not available, generate simulated data based on established relationships between environmental conditions, spray parameters, and application outcomes.

  - Preprocess and organize the data into model inputs and measurable spray outputs.

- Predictive ML model

  - Inputs may include wind speed/direction, temperature, humidity, machine speed, boom height, nozzle type, pressure, and flow rate.

  - Outputs may include predicted coverage, deposition, or another selected spray-performance metric.

  - Compare appropriate regression-based ML methods depending on dataset size and quality.

  - Begin with a baseline model and increase model complexity only if supported by the available data.

- Model evaluation

  - Separate available data into training and validation/testing sets.

  - Evaluate prediction accuracy using appropriate error metrics such as MAE or RMSE.

  - Test whether predicted spray outcomes change reasonably as environmental and operating parameters are varied.

- System integration

  - Provide model predictions to the parameter-selection component.

  - Compare predicted results with post-spray measurements.

  - Use prediction error as feedback for later recommendations or model recalibration.

Responsibilities and Contribution

Skills: Machine learning, data analysis, predictive modeling, and software development.

Collaboration Method: Teams/Microsoft, shared project documents, biweekly meetings, and shared code repository as needed.

Expected Deliverables:

- Prepared experimental/public/simulated dataset for modeling.

- Predictive ML model and model code.

- Model evaluation and validation results.

- Integration of the predictive model into the final system demonstration or simulation.

Model Inputs

| Type | Parameter | Why it matters |
| --- | --- | --- |
| Environment | Wind Speed | Major driver of off-target movement |
| Environment | Wind Direction | Determines direction of displacement |
| Environment | Temperature | Influences evaporation |
| Environment | Relative Humidity | Influences evaporation/droplet shrinkage |
| Machine | Travel Speed | Changes application rate and airflow around spray |
| Machine | Boom height | Changes flight time and therefore drift |
| Spray System | Nozzle type/size | Strongly determines flow, pattern, and droplet spectrum |
| Spray System | Pressure | Affect flow rate and droplet size |
| Spray System | Flow Rate | Determines spray volume/application rate |
| Spray System | Spray Angle | Affects pattern width/overlap |
| Intermediate | Droplet size | Critical link between nozzle/pressure and drift/deposition |
| Output | Target deposition | How much actually reached |
| Output | Coverage/uniformity | How well is target area covered |
| Output | Off-target deposition/ drift | How much spray is displaced? |

Parameter Prioritization

- Highest

  - Wind speed

  - Pressure

  - Nozzle type/size

  - Boom height

  - Travel speed

  - Why: strong influence, measurable, controllable or operationally important

- High

  - Droplet size

  - Flow right

  - Why: key intermediate variables connecting settings to drift/deposition

- High

  - Drift/ off target deposition, target deposition

  - Why: core project outputs

- Medium

  - Temperature, relative humidity

  - Why: important mainly through evaporation/droplet behavior

- Medium

  - Coverage/ uniformity

  - Why: valuable outcome but harder to define consistently

- Lower for V1

  - Spray angle, formulation properties, canopy structure, turbulence, droplet charge, etc

Outputs:

- Spray Quality: droplet size + pattern/uniformity

- Target performance: coverage + deposition

- Loss: off-target deposition/drift

Project Pieces

- Define the model inputs, outputs, and controllable parameters.

- Define the relationships/data used to simulate spray behavior

- Define the optimization logic that determines the “best” setting.

Simulation

INPUT CONDITIONS

Wind, pressure, speed, nozzle, boom height, etc.
->

SPRAY BEHAVIOR MODEL

->

PREDICTED WSP-LIKE RESULT

Coverage %, droplet density, distribution, size

->

PERFORMANCE ASSESSMENT

“good / poor coverage”

“too fine / drift prone”

“non-uniform pattern”

->

OPTIMIZER

Changes pressure / speed / etc

->

NEW PREDICTED WSP RESULT

WSP

- Create a small library of WSP examples and associate each image with known metrics

- Ex:

| Example | Coverage | Droplet Density | Size class | Uniformity |
| --- | --- | --- | --- | --- |
| A | 25% | 20 drops/sqcm | coarse | poor |
| B | 45% | 45 drops/sqcm | medium | moderate |
| C | 70% | 80 drops/sqcm | medium | good |
| D | 85% | 120 drops/sqcm | fine | good |

PARAMETERS

Wind Speed:

- Should be the ambient horizontal air velocity at or near spray-release height during application.

  - https://www.epa.gov/pesticides/mitigation-menu-measure-descriptions

- Category: Environmental

- Unit: meters / second

- Role: Continuous environmental input used to predict droplet displacement, off-target deposition, target deposition, and coverage behavior.

- Controllable: No

  - However can respond by changing controllable parameters (spray pressure, vehicle speed, boom height, apply/don’t apply)

- Measured

  - How should we measure

- Range

  - 0-7 m/s or 0-15 mph

  - Define greater than 15 mph as prohibiting application